Use Pages 1 & 2 as a reference to the Lab. It will become very helpful for learning the fundamentals before executing Lab 1.  Use Pages 3 & 4 to download the software for the following Labs.

Components

Click any photo to learn more about the component!

Breadboard

- Wires, Resistors, and LED’s are place in the holes, interconnected with the holes below/above.

- Split into two parts, not connected to one another.

LED On

LEDs

- The longer wire is the Anode and shorter is Cathode

- Power will be going through the long wire first to illuminate the LED.

Resistors

Resistor Symbol

Resistors are able to be placed in either direction.

100Ω - (Brown, Black, Brown, Gold)

Colour coordinated to display

the resistive value.

470Ω - (Yellow, Violet, Brown, Gold)

- Used to create current and

1kΩ - (Brown, Black, Red, Gold)

protect other components.

2.2kΩ - (Red, Red, Red, Gold)

Switches

- The switch is not allowing any power through

when it is not pressed.

- When pressed down, it allows for power to go

through.

Connected Wires

Arduino-Uno

Ground

Connect to Computer

Connect to Power after Programmed

Ground

Schematic Example:

Arduino Software

Windows Install Instructions

If your laptop or computer is running Windows:

- In your web browser, go to www.arduino.cc

- Under the software tab, click downloads

- Click “windows installer”

- Click “Just Download”

- If necessary, click “save file”

- Once the file has finished downloading, navigate to your downloads folder and double click “Arduino-1.8.8-windows.exe”.

- A warning may appear asking if you want “this app to make changes to your device”. Click yes.

- Click your way through the installer. Three times in the install process a window will open asking “would you like to install this device software”. Always click Install.

- Once the software is finished installing, go to the start menu and find “Arduino”.

- The first time you launch the software it’s going to ask if you want to let it through the firewall. Click Allow Access.

- The Arduino software is installed, you’re ready for your first Arduino program!

MacOS Install Instructions

If your laptop or computer is running MacOS:

- In your web browser, go to www.arduino.cc

- Under the software tab, click downloads

- Click “Mac OS X”.

- Click “Just download”

- Once the file has finished downloading, navigate to the downloads folder and move Arduino.app to the applications folder.

- Navigate to the applications folder and launch Arduino. The first time you start it a window will open asking if you really want to open. Click “Open”.

- The Arduino software is installed, you’re ready for your first Arduino program!